Task:

a) Design a voltage divider to fulfil below requirement. (Total: 15%)

i. Use a resistor and LDR only.

#include <Arduino.h>

void setup() {

pinMode(A0,INPUT);

Serial.begin(115200);

}

void loop() {

Serial.print("LDR: ");

Serial.print(analogRead(A0));

Serial.print('\n');

delay(100);

}

ii. Draw the circuit diagram and label the name and value. (5%)

iii. What is the ADC resolution of ESP8266? What is the range of ADC value? Show the calculation step. (5%)

iv. What is the value of voltage if the ADC value is 50. Show the calculation step. (5%)

b) Build a smart lighting system and fulfil below requirement. (Total: 25%)

i. Connect the light sensor and LED to ESP8266. Draw the circuit diagram and label the name, value and pin name. Then, take a clear photo. (No capture or screenshot) (5%)

ii. Code a program to print the value of brightness by percentage on serial monitor. (Recommend to use “hterm”) The format is “Brightness: xx%”. Capture the output and source code. (10%)

#include <Arduino.h>

int x = 450;

int y = 1023;

int temp =0;

float brightness = 0;

int temp2 =0;

void setup() {

pinMode(A0,INPUT);

pinMode(D1,OUTPUT);

Serial.begin(115200);

}

void loop() {

temp = analogRead(A0);

temp2 = map(temp, x, y, 0, 255);

analogWrite(D1,temp2);

brightness = (100-temp2*100/255);

Serial.print("Brightness:  ");

Serial.print(brightness);

Serial.print("%");

Serial.print('\n');

delay(100);

}

iii. Control the brightness of LED based on the environment. When the brightness is lower than 50%, turn on the LED. Otherwise, turn off. Capture the source code. (10%)

#include <Arduino.h>

int x = 450;

int y = 1023;

int temp =0;

float brightness = 0;

int temp2 =0;

void setup() {

pinMode(A0,INPUT);

pinMode(D1,OUTPUT);

Serial.begin(115200);

}

void loop() {

temp = analogRead(A0);

temp2 = map(temp, x, y, 0, 255);

brightness = (100-temp2*100/255);

if(brightness<50)

{

digitalWrite(D1,HIGH);//The LED turns ON in Dark.

}

else

{

digitalWrite(D1,LOW);//The LED turns OFF in Light.

}

Serial.print("Brightness:  ");

Serial.print(brightness);

Serial.print("%");

Serial.print('\n');

delay(100);

}

c) Code a program to read the temperature and humidity by DHT22 sensor.

(Total: 15%)

i. Draw the circuit diagram and label the name, value and pin number. (5%)

ii. Print the value of temperature and humidity to serial monitor. The format is “Temp: xxx°C, Humi: yyy%”. Capture the output and source code. (10%)

#include <Adafruit_Sensor.h>

#include <DHT.h>

#include <DHT_U.h>

#define DHTPIN 4     // Digital pin connected to the DHT sensor

// Feather HUZZAH ESP8266 note: use pins 3, 4, 5, 12, 13 or 14 --

// Pin 15 can work but DHT must be disconnected during program upload.

// Uncomment the type of sensor in use:

//#define DHTTYPE    DHT11     // DHT 11

#define DHTTYPE    DHT22     // DHT 22 (AM2302)

//#define DHTTYPE    DHT21     // DHT 21 (AM2301)

// See guide for details on sensor wiring and usage:

//   https://learn.adafruit.com/dht/overview

DHT_Unified dht(DHTPIN, DHTTYPE);

uint32_t delayMS;

void setup() {

Serial.begin(9600);

// Initialize device.

dht.begin();

Serial.println(F("DHTxx Unified Sensor Example"));

// Print temperature sensor details.

sensor_t sensor;

dht.temperature().getSensor(&sensor);

Serial.println(F("------------------------------------"));

Serial.println(F("Temperature Sensor"));

Serial.print  (F("Sensor Type: ")); Serial.println(sensor.name);

Serial.print  (F("Driver Ver:  ")); Serial.println(sensor.version);

Serial.print  (F("Unique ID:   ")); Serial.println(sensor.sensor_id);

Serial.print  (F("Max Value:   ")); Serial.print(sensor.max_value); Serial.println(F("°C"));

Serial.print  (F("Min Value:   ")); Serial.print(sensor.min_value); Serial.println(F("°C"));

Serial.print  (F("Resolution:  ")); Serial.print(sensor.resolution); Serial.println(F("°C"));

Serial.println(F("------------------------------------"));

// Print humidity sensor details.

dht.humidity().getSensor(&sensor);

Serial.println(F("Humidity Sensor"));

Serial.print  (F("Sensor Type: ")); Serial.println(sensor.name);

Serial.print  (F("Driver Ver:  ")); Serial.println(sensor.version);

Serial.print  (F("Unique ID:   ")); Serial.println(sensor.sensor_id);

Serial.print  (F("Max Value:   ")); Serial.print(sensor.max_value); Serial.println(F("%"));

Serial.print  (F("Min Value:   ")); Serial.print(sensor.min_value); Serial.println(F("%"));

Serial.print  (F("Resolution:  ")); Serial.print(sensor.resolution); Serial.println(F("%"));

Serial.println(F("------------------------------------"));

// Set delay between sensor readings based on sensor details.

delayMS = sensor.min_delay / 1000;

}

void loop() {

// Delay between measurements.

delay(delayMS);

// Get temperature event and print its value.

sensors_event_t event;

dht.temperature().getEvent(&event);

if (isnan(event.temperature)) {

Serial.println(F("Error reading temperature!"));

}

else {

Serial.print(F("Temp: "));

Serial.print(event.temperature);

Serial.print(F("°C"));

Serial.print(", ");

}

// Get humidity event and print its value.

dht.humidity().getEvent(&event);

if (isnan(event.relative_humidity)) {

Serial.println(F("Error reading humidity!"));

}

else {

Serial.print(F("Humi: "));

Serial.print(event.relative_humidity);

Serial.println(F("%"));

}

}

d) Create a web server on ESP8266. It includes below functions. (Total: 45%)

i. Show the real time temperature and humidity. (15%)

ii. Show the real time brightness value. (10%)

iii. Have a switch to control the LED. (10%)

iv. Good coding style. (5%)

v. Nice outlook of website. (5%)

When light turns on

When light turns off

// Import required libraries

#include <Arduino.h>

#include <ESP8266WiFi.h>

#include <Hash.h>

#include <ESPAsyncTCP.h>

#include <ESPAsyncWebServer.h>

#include <Adafruit_Sensor.h>

#include <DHT.h>

int x = 450;

int y = 1023;

int temp =0;

const char* PARAM_INPUT_1 = "output";

const char* PARAM_INPUT_2 = "state";

// Replace with your network credentials

const char* ssid = "noddy wifi";

const char* password = "55434050";

#define DHTPIN 5     // Digital pin connected to the DHT sensor

// Uncomment the type of sensor in use:

//#define DHTTYPE    DHT11     // DHT 11

#define DHTTYPE    DHT22     // DHT 22 (AM2302)

//#define DHTTYPE    DHT21     // DHT 21 (AM2301)

DHT dht(DHTPIN, DHTTYPE);

// current temperature & humidity, updated in loop()

float t = 0.0;

float h = 0.0;

float b = 0.0;

// Create AsyncWebServer object on port 80

AsyncWebServer server(80);

// Generally, you should use "unsigned long" for variables that hold time

// The value will quickly become too large for an int to store

unsigned long previousMillis = 0;    // will store last time DHT was updated

// Updates DHT readings every 10 seconds

const long interval = 10000;

const char index_html[] PROGMEM = R"rawliteral(

<!DOCTYPE HTML><html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1">

<link rel="stylesheet" href="https://use.fontawesome.com/releases/v5.7.2/css/all.css" integrity="sha384-fnmOCqbTlWIlj8LyTjo7mOUStjsKC4pOpQbqyi7RrhN7udi9RwhKkMHpvLbHG9Sr" crossorigin="anonymous">

<style>

html {

font-family: Arial;

display: inline-block;

margin: 0px auto;

text-align: center;

}

h2 { font-size: 3.0rem; }

p { font-size: 3.0rem; }

.units { font-size: 1.2rem; }

.dht-labels{

font-size: 1.5rem;

vertical-align:middle;

padding-bottom: 15px;

}

.switch {position: relative; display: inline-block; width: 120px; height: 68px}

.switch input {display: none}

.slider {position: absolute; top: 0; left: 0; right: 0; bottom: 0; background-color: #ccc; border-radius: 6px}

.slider:before {position: absolute; content: ""; height: 52px; width: 52px; left: 8px; bottom: 8px; background-color: #fff; -webkit-transition: .4s; transition: .4s; border-radius: 3px}

input:checked+.slider {background-color: #b30000}

input:checked+.slider:before {-webkit-transform: translateX(52px); -ms-transform: translateX(52px); transform: translateX(52px)}

</style>

</head>

<body>

<h2>ESP8266 DHT Server</h2>

<p>

<i class="fas fa-thermometer-half" style="color:#059e8a;"></i>

<span class="dht-labels">Temperature</span>

<span id="temperature">%TEMPERATURE%</span>

<sup class="units">&deg;C</sup>

</p>

<p>

<i class="fas fa-tint" style="color:#00add6;"></i>

<span class="dht-labels">Humidity</span>

<span id="humidity">%HUMIDITY%</span>

<sup class="units">percent</sup>

</p>

<p>

<span class="dht-labels">Brightness</span>

<span id="brightness">%BRIGHTNESS%</span>

<sup class="units">%</sup>

</p>

<h4>Output - GPIO 2</h4>

<label class="switch"><input type="checkbox" onchange="toggleCheckbox(this)" id="2" " + outputState(2) + "><span class="slider"></span></label>

</body>

<script>

setInterval(function ( ) {

var xhttp = new XMLHttpRequest();

xhttp.onreadystatechange = function() {

if (this.readyState == 4 && this.status == 200) {

document.getElementById("temperature").innerHTML = this.responseText;

}

};

xhttp.open("GET", "/temperature", true);

xhttp.send();

}, 10000 ) ;

setInterval(function ( ) {

var xhttp = new XMLHttpRequest();

xhttp.onreadystatechange = function() {

if (this.readyState == 4 && this.status == 200) {

document.getElementById("humidity").innerHTML = this.responseText;

}

};

xhttp.open("GET", "/humidity", true);

xhttp.send();

}, 10000 ) ;

setInterval(function ( ) {

var xhttp = new XMLHttpRequest();

xhttp.onreadystatechange = function() {

if (this.readyState == 4 && this.status == 200) {

document.getElementById("brightness").innerHTML = this.responseText;

}

};

xhttp.open("GET", "/brightness", true);

xhttp.send();

}, 10000 ) ;

function toggleCheckbox(element) {

var xhr = new XMLHttpRequest();

if(element.checked){

xhr.open("GET", "/update?output="+element.id+"&state=1", true);

}

else {

xhr.open("GET", "/update?output="+element.id+"&state=0", true);

}

xhr.send();

}

</script>

</html>)rawliteral";

String outputState(int output){

if(digitalRead(output)){

return "checked";

}

else {

return "";

}

}

String processor(const String& var){

//Serial.println(var);

if(var == "TEMPERATURE"){

return String(t);

}

if(var == "HUMIDITY"){

return String(h);

}

if(var == "BRIGHTNESS"){

return String(b);

}

if(var == "BUTTONPLACEHOLDER"){

String buttons = "";

buttons += "<h4>Output - GPIO 5</h4><label class=\"switch\"><input type=\"checkbox\" onchange=\"toggleCheckbox(this)\" id=\"5\" " + outputState(2) + "><span class=\"slider\"></span></label>";

return buttons;

}

return String();

}

void setup(){

// Serial port for debugging purposes

Serial.begin(115200);

dht.begin();

pinMode(2, OUTPUT);

digitalWrite(2, LOW);

pinMode(A0,INPUT);

// Connect to Wi-Fi

WiFi.begin(ssid, password);

Serial.println("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println(".");

}

// Print ESP8266 Local IP Address

Serial.println(WiFi.localIP());

// Route for root / web page

server.on("/", HTTP_GET, [](AsyncWebServerRequest *request){

request->send_P(200, "text/html", index_html, processor);

});

server.on("/temperature", HTTP_GET, [](AsyncWebServerRequest *request){

request->send_P(200, "text/plain", String(t).c_str());

});

server.on("/humidity", HTTP_GET, [](AsyncWebServerRequest *request){

request->send_P(200, "text/plain", String(h).c_str());

});

server.on("/brightness", HTTP_GET, [](AsyncWebServerRequest *request){

request->send_P(200, "text/plain", String(b).c_str());

});

// Send a GET request to <ESP_IP>/update?output=<inputMessage1>&state=<inputMessage2>

server.on("/update", HTTP_GET, [] (AsyncWebServerRequest *request) {

String inputMessage1;

String inputMessage2;

// GET input1 value on <ESP_IP>/update?output=<inputMessage1>&state=<inputMessage2>

if (request->hasParam(PARAM_INPUT_1) && request->hasParam(PARAM_INPUT_2)) {

inputMessage1 = request->getParam(PARAM_INPUT_1)->value();

inputMessage2 = request->getParam(PARAM_INPUT_2)->value();

digitalWrite(inputMessage1.toInt(), inputMessage2.toInt());

}

else {

inputMessage1 = "No message sent";

inputMessage2 = "No message sent";

}

Serial.print("GPIO: ");

Serial.print(inputMessage1);

Serial.print(" - Set to: ");

Serial.println(inputMessage2);

request->send(200, "text/plain", "OK");

});

// Start server

server.begin();

}

void loop(){

unsigned long currentMillis = millis();

if (currentMillis - previousMillis >= interval) {

// save the last time you updated the DHT values

previousMillis = currentMillis;

// Read temperature as Celsius (the default)

float newT = dht.readTemperature();

// Read temperature as Fahrenheit (isFahrenheit = true)

//float newT = dht.readTemperature(true);

// if temperature read failed, don't change t value

if (isnan(newT)) {

Serial.println("Failed to read from DHT sensor!");

}

else {

t = newT;

Serial.println(t);

}

// Read Humidity

float newH = dht.readHumidity();

// if humidity read failed, don't change h value

if (isnan(newH)) {

Serial.println("Failed to read from DHT sensor!");

}

else {

h = newH;

Serial.println(h);

}

float newB;

// Read brightness

temp = analogRead(A0);

newB = map(temp, x, y, 0, 100);

newB = (100 - newB);

// if LDR read failed, don't change b value

if (isnan(newB)) {

Serial.println("Failed to read from LDR sensor!");

}

else {

b = newB;

Serial.println(b);

}

}

}